D1 Admission Control — Python Model Specification v1.0

This specification defines the inputs, decision variables, objective function, constraints, functions, and assumptions required to reproduce the D1 optimization algorithm.

All Python variable and function names will remain consistent in the subsequent implementation.

1. Basic Configuration

| Parameter | Value | Description |
| --- | --- | --- |
| N_GUESTS | 6 | G1–G6 |
| RESOURCE_NAMES | See below | Eight resource classes |
| MISSION_TYPE | "survey" | Initial mission (assumed) |
| SOLVER_TIME_LIMIT | 30 | Maximum solver runtime (seconds) |
| SOLVER_RANDOM_SEED | 42 | Fixed random seed |
| MISSION_DURATION_DAYS | 14 | Mission duration |

RESOURCE_NAMES = [    "power_kw",    "cooling_kw",    "deck_space",    "mast_slots",    "mass_t",    "data_mbps",    "compute",    "response_time_s"]

These represent eight resource classes, not necessarily eight independent numerical variables.

2. Guest Parameters (G1–G6)

| Guest | power_modes_kw | mass_t | allowed_positions |
| --- | --- | --- | --- |
| G1 | Normal: 1.5 | 0.045 | Mast |
| G2 | Normal: 6; Winch: 20 | 3.5 | Bay 2, Half Bay 1 |
| G3 | Charging: 25; Standby: 1 | 5.5 | Bay 1 |
| G4 | Charging: 35; Normal: 10; Standby: 2 | 1.04 | Hangar |
| G5 | Normal: 45; Low: 8 | 3.025 | Bay 2, Half Bay 1; antenna on Mast / Hangar Roof |
| G6 | Normal: 12; Low: 4 | 0.5 | Bay 2 (assumed) |

G6 parameters are hypothetical and can be replaced with the team's final design.

Each Guest will contain the following fields:

guest_id, name, power_modes_kw, mass_t, preferred_positions, allowed_positions, requires, excludes, part_of, safety_critical, installation.

Important: G2's winch operation and G3's charging cycle have specific durations and operating conditions. They cannot be treated as freely interchangeable power modes.

3. Ship Resource Parameters (8V)

| Python Parameter | Initial Value | Description |
| --- | --- | --- |
| power_capacity_kw | 200 | Normal cruising power reserve |
| degraded_power_kw | 100 | Reserve with one generator offline |
| power_line_capacity_kw | 150 | Capacity per mission power line |
| cooling_capacity_12c_kw | 160 | Cooling capacity at 12°C |
| cooling_capacity_30c_kw | 120 | Cooling capacity at 30°C |
| cooling_degradation_factor | 0.6 | Remaining capacity after cooling failure |
| mass_capacity_t | 10 | Available mass budget |
| stability_capacity_t_eq | 12 | Stability-equivalent budget |
| mast_total_slots | 3 | Total mast positions |
| mast_occupied_slots | 2 | Positions already occupied |
| satcom_capacity_mbps | 8 | Assumed upper-end SATCOM capacity |
| backbone_capacity_mbps | 10000 | Capacity per 10 Gbit/s backbone link |
| compute_allocatable_fraction | 0.8 | Compute fraction available to Guests |

Additional parameters are required for deck dimensions, placement compatibility, individual power lines, communication links, and peak-demand durations.

4. Optimization Decision Variables

| Python Variable | Type | Description |
| --- | --- | --- |
| installed[i] | Binary | Whether Guest \(i\) is physically installed |
| mode_selected[i,m] | Binary | Whether Guest \(i\) operates in mode \(m\) |
| position_selected[i,p] | Binary | Whether Guest \(i\) occupies position \(p\) |
| power_line_selected[i,l] | Binary | Whether Guest \(i\) uses power line \(l\) |
| action_selected[i,a] | Binary | Whether an adjustment action \(a\) is selected |

Physical installation and operational status must be modeled separately.

For example, turning G5 off does not automatically free its deck space or mass budget.

5. Mission Weights and Capability Values

The following values are initial modeling assumptions, except where explicitly provided by the challenge.

| Guest | mission_weights (Survey) | Normal Capability | Reduced Capability |
| --- | --- | --- | --- |
| G1 | 1 | 1.0 | Not defined |
| G2 | 5 | 1.0 | Standby: 0 (assumed) |
| G3 | 10 | 1.0 | Standby: 0 (assumed) |
| G4 | 1 | 1.0 | Standby: 0 (assumed) |
| G5 | 1 | 1.0 | Low: 0.3 |
| G6 | 2 | 1.0 | Low: 0.5 (assumed) |

For G5, the challenge explicitly states that Low Mode causes a 70% capability loss.

All assumed weights and capability values must be documented in the Physical Account.

6. Objective Function

Maximize Total Mission Utility

\[ \boxed{ \max U=\sum_{i=1}^{6}\sum_{m\in M_i} w_i c_{im}x_{im} } \]

Where:

- \(w_i\): Mission priority weight of Guest \(i\).

- \(c_{im}\): Capability retained by Guest \(i\) in mode \(m\), between 0 and 1.

- \(x_{im}\): Binary variable indicating whether mode \(m\) is selected.

- \(M_i\): Set of available operating modes for Guest \(i\).

Python equivalents:

mission_weights[i], capability[i][m], mode_selected[i,m].

The new Guest's requested admission, permitted removal actions, and safety rules must also be included in the model. Otherwise, the optimizer might maximize utility simply by refusing the new Guest.

7. Constraint Functions

| Python Function | Description |
| --- | --- |
| add_mode_constraints() | Operating mode and installation consistency |
| add_power_constraints() | Continuous power, peak power, power-line limits |
| add_cooling_constraints() | Heat rejection, power–heat coupling, cooling degradation |
| add_placement_constraints() | Deck, mast, hangar, and placement compatibility |
| add_mass_constraints() | Total mass and stability budget |
| add_data_constraints() | Communication-link capacity and availability |
| add_compute_constraints() | CPU, RAM, accelerator allocation |
| add_time_constraints() | Admission response time and operation durations |
| add_ship_state_constraints() | Speed, course, noise, deck, emission states |
| add_operational_constraints() | Requires, Excludes, Part-of, safety rules |
| add_installation_constraints() | Port and underway installation restrictions |
| add_mission_envelope_constraints() | Feasibility under normal and degraded mission conditions |

All applicable hard constraints must be satisfied before a configuration is accepted.

8. Main Workflow Functions

validate_manifest()load_guest_data()load_ship_budget()load_mission_envelope()generate_candidates()build_model()solve_model()classify_admission()explain_decision()

Execution workflow:

Load Inputs

Validate Guest Manifest

Generate Candidate Configurations

Build Constraints

Optimize with OR-Tools CP-SAT

Classify Admission

YES / YES-IF / NO / NOT-NOW

Generate Explanation + Binding Constraints

9. Missing Parameters and Assumptions

The following parameters are not fully specified in the challenge and require explicit assumptions or team input:

- CPU, RAM, and accelerator requirements for each Guest and operating mode.

- Complete G6 specifications, including communication, installation requirements, capability, and dependencies.

- G2 standby power and other unspecified operating modes.

- Capability retention values for modes not quantified in Annex B.

- Numerical mission-priority weights.

- Assignment of Guests to individual power lines.

- Scheduling rules for overlapping peak loads and charging cycles.

- Capability losses and ranking criteria for displacing installed Guests.

All invented values must be documented in the Physical Account (Annex C).

Implementation status: This is a reproducible design specification for the initial D1 prototype, not yet a fully specified executable model. The main structure and naming conventions are established, while the missing parameters, detailed placement rules, and temporal constraints still need to be finalized or explicitly assumed in the Python implementation.